PC TOOL FOR THE XA137

- Install the software onto your PC and run the software

- Set up the machine with till roll and switch ON

- Connect machine via USB

- Select COM port – press CONNECT (COM4)

- Click on the different options as required

- Setting – for receipt header and tail (footer). First UPLOAD then add all the details that you wish to see on the machine –– then DOWNLOAD – then switch OFF the machine at the ON/OFF switch and when back on press EN on the machine it will then save that information to the machine.

OPTIONS - (PRT – print on receipt), (2H – Double width and height), (Mid – Centre text)

*Do not forget to switch on Print RCP on that page*

*Change tax rates on the settings page you have 4 available rates*

- ReadDEPT and ReadPLU will bring information from the machine into the software

- DEPT will open a list of the departments you can change

Click on READ departments to show the DEPT list

Add name then price, if discount is allowed and you can set tax rate – click onto the next unused dept when finished this saves the list above for you and press WRITE DEPT for the information to go to the machine. Example for prices £0.50 is written as 50, £2.00 is 200, £4.95 is 495. If you want an OPEN price leave the price as 0. When you have finished the list press WriteDept

- PLU will create a list of PLUs – click in the box to add details for that specific item – (more details below if need a breakdown of all the columns)

- ONLINE STATUS appears when it is uploading. The machine will then take you back to the settings page. Re-enter registration mode.

IMPORT and EXPORT to CSV file

You can use the machine using the EXPORT and IMPORT file feature – firstly press on READDept and then OUTPUT file and give the file a name Ex. DEPT1.  Next go to READ Plu and then name that file ex. PLU1. Now I have created 2 spreadsheets for my products.

In the DEPT spread sheet – Enter details as required

- ID is SKU (sometimes auto generated)

- Name – type department name

- Price – enter price or leave as 0 for open price

- Discount – if allowed enter TRUE (in caps)

- TAX – all are FALSE as, change to TRUE if tax level is required for that item

SAVE the spreadsheet – then Close CSV file before you import!

Press WriteDept to send to the machine

In PLU spreadsheet – Enter details as required

- ID – you may need to create a SKU (a number different for each product)

- No – number of the PLU

- Name – name of item

- Price – enter price (PLU must have a price)

SAVE and CLOSE spreadsheet

IMPORT file and then press WritePLU to send the details to the machine

RECORD/SALES

When you are connected you can upload the sales data from the machine to the software – press RECORD

This will generate a table with all sales on

You can then OUTPUT file to csv – this file can then be printed etc

LOGO

Click on Logo and it will say what the bitmap sizing should be – (W 384 and H200)

Create the logo with these size parameters and import the logo from a file saved on your computer

It will appear under the icons when it is uploaded to the software

Change within SETTINGS and tick PRINT LOGO and press DOWNLOAD

Turn the machine OFF then ON and when you make a sale the logo should be at the top of the receipt header – you do not lose any other information from the receipt.

SD CARD – NOT FOR USE WITH A PC

The SD card slot is to the right-hand side of the machine – located by the ON/OFF switch.

In settings list use arrow keys down and locate -

ECR data to SD card – this will allow you to save the date from this machine.

SD card data to ECR – this will allow you to upload information from the SD card to the new machine.

TIPS

- If the receipt header does not move to the machine – re-press download and then turn the machine on/off again – try producing a receipt or doing a config report to show changes

- Sometimes the machine may need you to press CONNECT again especially if doing imports etc.

- When you have set tax rates they will show on the receipt as – TAX 1=A, TAX 2=B